GUJRAAT STATE ELECTRICITY TRANSMISSION CORPORATION LIMITED

(GSETCL - A Government of Gujraat Undertaking)

TENDER DOCUMENT   Design, Engineering, Supply, Erection, Testing and Commissioning (Turnkey) of a new 132kV/33kV, 2x40 MVA Grid Substation at Morbi, Gujraat

| Field | Details |
| --- | --- |
| Tender Reference No. | GSETCL/SS-MORBI/2026/EPC-021 |
| Issuing Authority | Gujraat State Electricity Transmission Corporation Limited (GSETCL) |
| Mode of Tendering | Two-Envelope System (Technical Bid + Price Bid), e-Procurement Portal |
| Tender Fee | ₹15,000 (Non-refundable) |
| Earnest Money Deposit (EMD) | ₹1,20,00,000 (Refundable, or 2% of quoted Bid Value, whichever is higher) |
| Bid Validity | 150 days from the date of Price Bid opening |
| Date of Issue | 05-Jan-2026 |
| Pre-Bid Meeting | 16-Jan-2026, 11:00 AM, GSETCL Corporate Office, Vadodara |
| Last Date for Submission | 20-Feb-2026, 5:00 PM (IST) |
| Technical Bid Opening | 21-Feb-2026, 11:00 AM (IST) |
| Price Bid Opening | Intimated to technically qualified bidders only |

# 1. Scope of Work

The Contractor shall carry out complete turnkey Engineering, Procurement and Construction (EPC) of a new 132kV/33kV, 2x40 MVA Grid Substation, including 132kV Air-Insulated Switchyard (AIS) with 4 bays (2 line bays, 1 transformer bay spare-provisioned, 1 bus coupler), 33kV indoor VCB switchgear with 12 feeder panels, associated civil works, control room building, and interconnecting 132kV transmission line termination, on the terms set out in this Tender Document, covering:

- Detailed engineering, design basis report, and drawings (GA, SLD, protection scheme, earthing, cable schedule)

- Supply of 2 x 40 MVA, 132/33kV power transformers with OLTC

- Supply and erection of 132kV AIS equipment (CBs, CTs, PTs, isolators, lightning arresters, bus-bar structures)

- Supply and erection of 33kV indoor VCB switchgear panels (12 feeders)

- Numerical protection & control system, SCADA/RTU integration with State Load Despatch Centre (SLDC)

- Control & Relay Panels (CRP), DC battery system, AC/DC distribution boards

- Civil works: control room building, cable trenches, equipment foundations, boundary wall, internal roads

- Earthing & lightning protection system as per IS 3043

- Testing, pre-commissioning checks, and commissioning with State grid

- Comprehensive Operation & Maintenance support and defect liability for 24 months post-commissioning

# 2. Eligibility Criteria (Pre-Qualification)

| S.No. | Criteria | Requirement |
| --- | --- | --- |
| EC-1 | Average Annual Turnover | Minimum ₹50,00,00,000 (₹50 Crore) in each of the last 3 financial years (FY 2022-23 to 2024-25), CA-certified |
| EC-2 | Relevant Experience | Successful commissioning of at least 3 substations of 132kV or above voltage class in the last 7 years, OR 1 substation of 220kV or above |
| EC-3 | Net Worth | Positive net worth as per latest audited financial statement |
| EC-4 | Quality Certifications | Valid ISO 9001:2015, ISO 14001:2015 and ISO 45001:2018 certificates |
| EC-5 | Statutory Registrations | Valid PAN, GST Registration, EPFO and Electrical Contractor License (Grade A) |
| EC-6 | OEM Approval | Power transformers and 132kV circuit breakers must be sourced from GSETCL Approved Vendor List (AVL), Rev. 14 |
| EC-7 | Blacklisting Declaration | Self-certified, notarized undertaking of no blacklisting/debarment |
| EC-8 | EMD Submission | EMD of ₹1,20,00,000 as Bank Guarantee, valid 210 days |

# 3. Technical Requirements

| S.No. | Parameter | Minimum Requirement |
| --- | --- | --- |
| TR-1 | Power Transformer Rating | 2 x 40 MVA, 132/33kV, ONAN/ONAF, Delta-Star, with OLTC ±10x1.5% |
| TR-2 | Transformer Temperature Rise | Top oil ≤ 45°C, winding ≤ 50°C over ambient, per IS 2026 |
| TR-3 | 132kV Circuit Breaker | SF6, 3-pole, rated 2000A, 40kA for 3 sec, BIL 650kV |
| TR-4 | 132kV CT/PT | Class 0.2S metering, Class 5P20 protection, per IS 2705/3156 |
| TR-5 | 33kV Switchgear | Indoor VCB, metal-clad, rated 1250A, 25kA for 3 sec |
| TR-6 | Protection System | Numerical relays (IEC 61850 compliant) for differential, distance, O/C, E/F protection |
| TR-7 | SCADA/RTU | IEC 60870-5-104 protocol, integration with SLDC within 30 days of commissioning |
| TR-8 | Earthing System | Grid resistance ≤ 1 ohm, as per IS 3043 |
| TR-9 | Battery System | 220V DC, 300 AH VRLA, with dual battery chargers |
| TR-10 | Defect Liability Period | 24 months from date of commissioning |

# 4. Commercial Requirements

- Prices on firm, fixed-price, turnkey basis (FOR Site) inclusive of all duties except GST, shown separately.

- Performance Bank Guarantee (PBG) of 5% of Contract Value within 15 days of LOA, valid till expiry of DLP.

- Payment strictly per Milestone Payment Schedule; no mobilization advance beyond 10% against Bank Guarantee.

- Prices firm throughout contract period except statutory tax variation.

# 5. Evaluation Criteria

Evaluation shall proceed in three sequential stages: (1) Eligibility Screening (Compliant/Non-Compliant), (2) Technical Evaluation out of 100 marks with a qualifying minimum of 60, and (3) Price Evaluation of technically qualified bidders only, with L1 determined on the Total Evaluated Price after accounting for any normalization for deviations.

| Criteria | Max Marks |
| --- | --- |
| Relevant Project Experience (132kV+ substations) | 25 |
| Technical Design & Equipment Compliance (Section 3) | 25 |
| OEM Credentials & Approved Vendor List Compliance | 15 |
| Execution Methodology, Schedule & QA/QC Plan | 20 |
| Manpower, Plant & Machinery Deployment | 10 |
| HSE Plan & Past Safety Record | 5 |

# 6. Milestones & Payment Schedule

| Milestone | Deliverable | Timeline (from LOA) |
| --- | --- | --- |
| M1 | Mobilization, PBG submission, Insurance | T + 15 days |
| M2 | Design/drawing approval by GSETCL | T + 45 days |
| M3 | Transformer & 132kV equipment delivery to site | T + 180 days |
| M4 | Civil works completion (control room, foundations) | T + 150 days |
| M5 | Switchyard erection complete | T + 240 days |
| M6 | 33kV switchgear, CRP, SCADA installation | T + 260 days |
| M7 | Testing & pre-commissioning checks | T + 280 days |
| M8 | Commissioning with State grid | T + 300 days (10 months) |
| M9 | Defect Liability Period | T + 300 to T + 1020 days |

# 7. General Terms & Conditions

- Bid Validity: 150 days from Price Bid opening.

- Completion Period: 10 months (300 days) from LOA date, achieving commissioning with State grid.

- Liquidated Damages: 0.5% of Contract Value per week of delay, capped at 10%.

- Defect Liability Period: 24 months from commissioning.

- Dispute Resolution: Arbitration under the Arbitration & Conciliation Act, 1996; seat at Vadodara.

- Insurance: Contractor's All Risk (CAR) and Third-Party Liability Insurance for full contract duration.

- GSETCL reserves the right to accept/reject any bid and waive minor infirmities at its discretion.

# 8. Bid Submission Requirements

Bids shall be submitted online in two envelopes (Technical Bid without price information, and Price Bid) via the GSETCL e-Procurement portal, complete with all eligibility documents, technical proposal, methodology, QA/QC & HSE plans, manpower and equipment deployment plans, and the priced Bill of Quantities (BOQ) in the prescribed format. All pages must be digitally signed by the authorized signatory. Bids received after the due date/time shall be summarily rejected.